ANEXA II

MNEMONICELE INSTRUCŢIUNILOR PROCESORULUI 80X86

AAA	ASCII Adjust after Addition

AAD	ASCII Adjust AX before Division (second byte is divisor)

AAM	ASCII Adjust AX after Multiply (second byte is divisor)

AAS	ASCII Adjust AL after Subtraction

ADC	Integer Add with carry

ADD	Integer ADDition

AND	Logical AND

ARPL	Adjust RPL field of selector (286+)

BOUND	Check array index against BOUNDs (186+)

BSF	Bit Scan forward (386+)

BSR	Bit Scan reverse (386+)

BSWAP	Byte SWAP (486+)

BT	Bit Test (386+)

BTC	Bit Test and Complement (386+)

BTR	Bit Test and Reset (386+)

BTS	Bit Test and Set (386+)

CALL	CALL subroutine

CBW	Convert Byte to Word	(AL --> AX)

CWDE	Convert Word to Dword (386+)	(AX --> EAX)

CWD	Convert Word to Double	(AX --> DX:AX)

CDQ	Convert Double to Quad	(EAX --> EDX:EAX)

CLC	CLear the Carry flag

CLD	CLear the Direction flag (set to forward direction)

CLI	CLear the Interrupt flag (disable interrupts)

CLTS	CLear Task Switched flag in CR0 (286+)

CMC	Complement carry flag

CMP	CoMPare two operands

- CMPS 	CoMPare String operands

- CMPSB 	CoMPare String operands - Byte

- CMPSW 	CoMPare String operands - Word

CMPSD	CoMPare String operands - Double

CMPXCHG	CoMPare and EXCHanGe (486+)

CMPXCHG8B	CoMPare and EXCHanGe 8 Bytes (Pentium+)

CPUID	CPU IDentification (Pentium+)

DAA	Decimal Adjust AL after Addition

DAS	Decimal Adjust AL after Subtraction

DEC	DECrement

DIV	Unsigned DIVide

ENTER	Make stack frame for procedure parameters (186+)

ESC	ESCape

HLT	HaLT

IDIV	SIgned DIVide

IMUL	SIgned MULtiply

IN	INput from port

INC	INCrement

- INS 	INput from port to String

- INSB 	INput from port to String - Byte

INSW 	INput from port to String - Word

INSD	INput from port to String - Double

INT	Call INTerrupt procedure

INTO	Call INTerrupt procedure if Overflow

INVD	INValidate Data cache (486+)

INVLPG	INValidate TLB entry (486+)

IRET	RETurn from Interrupt

IRETD	32-bit RETurn from Interrupt (386+)

JA	Jump if Above

JAE	Jump if Above Or Equal

JB	Jump if Below

JBE	Jump if Below Or Equal

JC	Jump if Carry

JCXZ	Jump if CX = 0

JE	Jump if Equal

JECXZ	Jump if ECX = 0

JG	Jump if Greater

JGE	Jump if Greater Or Equal

JL	Jump if Less

JLE	Jump if Less Or Equal

JMP	Unconditional JuMP

JNA	Jump if Not Above

JNAE	Jump if Not Above Or Equal

JNB	Jump if Not Below

JNBE	Jump if Not Below Or Equal

JNC	Jump if Not Carry

JNE	Jump if Not Equal

JNG	Jump if Not Greater

JNGE	Jump if Not Greater Or Equal

JNL	Jump if Not Less

JNLE	Jump if Not Less Or Equal

JNO	Jump if Not Overflow

JNP	Jump if No Parity (odd)

JNS	Jump if Not Sign

JNZ	Jump if Not Zero

JO	Jump if Overflow

JP	Jump if Parity (even)

JPE	Jump if Parity Even

JPO	Jump if Parity Odd

JS	Jump if Sign

JZ	Jump if Zero

LAHF	Load Flags into AH

LAR	Load Access Rights byte (286+)

LSS	Load Stack Segment and offset

LEA	Load Effective Address

LGDT	Load Global Descriptor Table register (286+)

LIDT	Load Interrupt Descriptor Table register (286+)

LLDT	Load Local Descriptor Table register (286+)

LMSW	Load Machine Status Word (286+)

LOCK	LOCK bus on next instruction (prefix)

LODS	LOaD String operand

LODSB	LOaD String operand - Byte

LODSW	LOaD String operand - Word

LODSD	LOaD String operand - Double

LOOP	LOOP control with CX counter

LOOPE	LOOP while Equal

LOOPZ	LOOP while Zero

LOOPNE	LOOP while Not Equal

LOOPNZ	LOOP while Not Zero

LSL	Load Segment Limit (286+)

LTR	Load Task Register (286+)

MOV	MOVe data

MOVS	MOVe data from String to string

MOVSB	MOVe data from String to string - Byte

MOVSW	MOVe data from String to string - Word

MOVSD	MOVe data from String to string - Double

MOVSX	MOVe with Sign-eXtend (386+)

MOVZX	MOVe with Zero-eXtend (386+)

MUL	Unsigned MULtiply

NEG	Two's complement NEGation

NOP	No OPeration

NOT	One's complemenT Negation

OR	Logical inclusive OR

OUT	OUTput to port

OUTS	OUTput String to port

OUTSB	OUTput String to port - Byte

OUTSW	OUTput String to port - Word

OUTSD	OUTput String to port - Double

POP	POP a word/dword from the stack

POPA	POP All (186+)

POPAD	POP All Double (386+)

POPF	POP Flags

POPFD	POP Flags Double (386+)

PUSH	PUSH a word/dword to the stack

PUSHA	PUSH All (186+)

PUSHAD	PUSH All Double (386+)

PUSHF	PUSH Flags

PUSHFD	PUSH Flags Double (386+)

RCL	Rotate bits Left with CF

RCR	Rotate bits Right with CF

ROL	ROtate bits Left

ROR	ROtate bits Right

RDMSR	ReaD from Model Specific Register (Pentium+)

REP	REPeat string operation

REPE	REPeat while Equal (or zero) string operation

REPNE	REPeat while Not Equal (or not zero) string operation

RET	RETurn from procedure

RETN	RETurn Near

RETF	RETurn Far

RSM	Resume from System Management mode (Pentium+)

SAL	Shift Arithmetic Left

SAR	Shift Arithmetic Right

SHL	SHift Left (same as SAL)

SHR	SHift Right

SAHF	Store AH into Flags

SBB	Integer SuBtraction with Borrow

SCAS	SCAn String data

SCASB	SCAn String data - Byte

SCASW	SCAn String data - Word

SCASD	SCAn String data - Double

SET	Set byte to 1 on condition else set to 0 (386+)

SETA	SET byte to 1 if Above

SETAE	SET byte to 1 if Above Or Equal

SETB	SET byte to 1 if Below

SETBE	SET byte to 1 if Below Or Equal

SETC	SET byte to 1 if Carry

SETE	SET byte to 1 if Equal

SETG	SET byte to 1 if Greater

SETGE	SET byte to 1 if Greater Or Equal

SETL	SET byte to 1 if Less

SETLE	SET byte to 1 if Less Or Equal

SETNA	SET byte to 1 if Not Above

SETNAE	SET byte to 1 if Not Above Or Equal

SETNB	SET byte to 1 if Not Below

SETNBE	SET byte to 1 if Not Below Or Equal

SETNC	SET byte to 1 if Not Carry

SETNE	SET byte to 1 if Not Equal

SETNG	SET byte to 1 if Not Greater

SETNGE	SET byte to 1 if Not Greater Or Equal

SETNL	SET byte to 1 if Not Less

SETNLE	SET byte to 1 if Not Less Or Equal

SETNO	SET byte to 1 if Not Overflow

SETNP	SET byte to 1 if No Parity (odd)

SETNS	SET byte to 1 if Not Sign

SETNZ	SET byte to 1 if Not Zero

SETO	SET byte to 1 if Overflow

SETP	SET byte to 1 if Parity (even)

SETPE	SET byte to 1 if Parity Even

SETPO	SET byte to 1 if Parity Odd

SETS	SET byte to 1 if Sign

SETZ	SET byte to 1 if Zero

SGDT	Store Global Descriptor Table register (286+)

SIDT	Store Interrupt Descriptor Table register (286+)

SHLD	Double precision SHift Left (386+)

SHRD	Double precision SHift Right (386+)

SLDT	Store Local Descriptor Table register (286+)

SMSW	Store Machine Status Word (286+)

STC	SeT the Carry flag

STD	SeT Direction flag (set to reverse string direction)

STI	SeT Interrupt flag (enable)

STOS	STOre String data

STOSB	STOre String data - Byte

STOSW	STOre String data - Word

STOSD	STOre String data - Double

STR	Store Task Register (286+)

SUB	Integer SUBtraction

TEST	Logical compare

VERR	VERify a segment for Reading (286+)

VERW	VERify a segment for Writing (286+)

WAIT	WAIT for co-processor

WBINVD	Write-Back and INValidate Data cache (486+)

WRMSR	WRite to Model Specific Register (PENTIUM+)

XADD	EXchange and ADD (486+)

XCHG	EXcHanGe register/memory with register

XLAT	Table Look-up translAtion

XLATB	Table Look-up translAtion - Byte

XOR	Logical eXclusive OR