ANEXA V

MNEMONICELE INSTRUCŢIUNILOR COPROCESORULUI 80X87

FABS	ABSolute value

FADD	Floating point ADD

FADDP	Floating point ADD and Pop

FBLD	LoaD BCD

FBSTP	STore BCD and Pop

FCHS	CHange Sign

FCLEX	CLear EXceptions

FNCLEX	CLear EXceptions, No wait

FCOM	Floating point COMpare

FCOMP	Floating point COMpare and Pop

FCOMPP	Floating point COMpare and PoP twice

FCOS	Floating point COSine (387+)

FDECSTP	DECrement floating point STack Pointer

FDISI	DISable Interrupts (8087 only, others do fnop)

FNDISI	DISable Interrupts, No wait (8087 only, others do fnop)

FDIV	Floating DIVide

FDIVP	Floating DIVide and Pop

FDIVR	Floating DIVide Reversed

FDIVRP	Floating DIVide Reversed and Pop

FENI	ENable Interrupts (8087 only, others do fnop)

FNENI	ENable Interrupts, No wait (8087 only, others do fnop)

FFREE	FREE register

FIADD	Integer ADD

FICOM	Integer COMpare

FICOMP	Integer COMpare and Pop

FIDIV	Integer DIVide

FIDIVR	Integer DIVide Reversed

FILD	LoaD Integer

FIMUL	Integer MULtiply

FINCSTP	INCrement floating point STack Pointer

FINIT	INITialize floating point processor

FNINIT	INITialize floating point processor, No wait

FIST	STore Integer

FISTP	STore Integer and Pop

FISUB	Integer SUBtract

FISUBR	Integer SUBtract reversed

FLD	Floating point LoaD

FLDZ	LoaD constant onto stack, 0.0 Zero

FLD1	LoaD constant onto stack, 1.0

FLDL2E	LoaD constant onto stack, Logarithm base 2 (E)

FLDL2T	LoaD constant onto stack, Logarithm base 2 (10 Ten)

FLDLG2	LoaD constant onto stack, Logarithm base 10 Ten (2)

FLDLN2	LoaD constant onto stack, natural Logarithm (2)

FLDPI	LoaD constant onto stack, pi (3.14159...)

FLDCW	LoaD Control Word

FLDENV	Load ENVironment state

FMUL	Floating point MULtiply

FMULP	Floating point MULtiply and Pop

FNOP	No OPeration

FPATAN	Partial ArcTANgent

FPREM	Partial REMainder

FPREM1	Partial REMainder (IEEE compatible, 387+)

FPTAN	Partial TANgent

FRNDINT	RouND to INTeger

FRSTOR	ReSTORe saved state

FSAVE	SAVE FPU state

FSAVEW	SAVE FPU state, 16-bit format (387+) - Word

FSAVED	SAVE FPU state, 32-bit format (387+) - Double

FSCALE	SCALE by factor of 2

FSETPM	SET Protected Mode (287 only, 387+ = fnop)

FSIN	SINe (387+)

FSINCOS	SINe and COSine (387+)

FSQRT	SQuare RooT

FST	Floating point STore

FSTP	Floating point STore and Pop

FSTCW	Store Control Word

FNSTCW	Store Control Word, No wait

FSTENV	Store FPU ENVironment

FSTENVW	Store FPU ENVironment, 16-bit format (387+) - Word

FSTENVD	Store FPU ENVironment, 32-bit format (387+) - Double

FNSTENV	Store FPU ENVironment, No wait

FNSTENVW	Store FPU ENVironment, No wait, 16-bit format (387+) - Word

FNSTENVD	Store FPU ENVironment, No wait, 32-bit format (387+) - Double

FSTSW	Store Status Word

FNSTSW	Store Status Word, no wait

FSUB	Floating point SUBtract

FSUBP	Floating point SUBtract and Pop

FSUBR	Floating point Reverse SUBtract

FSUBRP	Floating point Reverse SUBtract and Pop

FTST	Floating point TeST for zero

FUCOM	Unordered floating point COMPare (387+)

FUCOMP	Unordered floating point COMPare and Pop (387+)

FUCOMPP	Unordered floating point COMPare and Pop twice (387+)

FWAIT	WAIT while FPU is executing

FXAM	EXAMine condition flags

FXCH	EXCHange floating point registers

FXTRACT	EXTRACT exponent and significand